# ENVIRONMENTAL IMPACT ASSESSMENT (EIA) - DETAILED GUIDE

# Thailand Data Center Development

---

# QUICK REFERENCE

---

# SECTION 1: LEGAL FRAMEWORK AND REQUIREMENTS

## National Environmental Quality Act B.E. 2535 (1992)

**Key Provisions:**

Section 5: Requires EIA for "projects that may affect natural resources and environment"

Section 7: ONEP determines which projects require EIA

Section 8: EIA mandatory before any construction or land alteration

Section 21: Violators face penalties of THB 100K-5M and possible imprisonment

**ONEP Notification on Projects Requiring EIA:**

Lists specific project types requiring EIA

Data centers appear in Schedule as "computer service centers" or "data centers"

Threshold: Generally projects >500 kW or certain locations

## When Full EIA Required vs. Notification

**Full EIA Required (If ANY True):**

Data center capacity >500 kW

Location in environmentally sensitive area:

- Wetlands or coastal area

- National parks or protected forests

- Watershed protection zones

- Flood-prone areas requiring special mitigation

Proximity to residential areas (<500m)

Using significant water resources (>1,000 m³/day expected usage)

In provinces with stricter environmental standards

**EIA Notification Only (No Full EIA, but notification required):**

Small data centers <500 kW capacity

Non-sensitive location

Low environmental impact

Distance from residential areas

**Practical Recommendation:** Almost all commercial data centers (>1 MW) in Thailand should plan for full EIA

## EIA Screening Process

**Step 1: Project Description Submission**

Submit brief project description to ONEP

Include:

- Location and site map

- Project description and capacity

- Environmental context (proximity to protected areas, residences, waterways)

- Expected environmental impacts (initial assessment)

**Step 2: ONEP Screening Determination (2-3 weeks)**

ONEP decides if full EIA required

Possible outcomes:

- **Full EIA Required:** Detailed study mandatory

- **Notification Only:** Brief environmental statement sufficient

- **Exemption:** No EIA needed (rare for data centers)

**Get ONEP Screening Certificate:** Required before proceeding with EIA study or construction

---

# SECTION 2: FULL EIA PROCESS - DETAILED TIMELINE

## Phase 1: Screening (Weeks 1-4)

**Activity 1.1: Prepare Project Description**

Document location (address, GPS coordinates)

Site plan showing:

- Building footprint

- Property boundaries

- Access roads

- Utilities (power, water, telecom)

- Buffer zones/setbacks

- Surrounding land use (within 1 km radius)

Project specifications:

- Data center capacity (kW, MW)

- Building size (square meters)

- Expected employees (Thai and foreign)

- Operating hours

- Construction duration

- Investment amount

Environmental context:

- Proximity to protected areas (national parks, wetlands, etc.)

- Distance to residential areas

- Proximity to waterways

- Current land use classification

- Flood risk assessment

- Air quality baseline

**Activity 1.2: Submit Screening Request to ONEP**

**ONEP Screening Submission:**

Address: Office of Natural Resources and Environmental Policy & Planning (ONEP)

- Ministry of Natural Resources and Environment

- 60/1 Rama VI Rd, Phayathai, Bangkok 10400

- Phone: 0-2535-8800

- Website: www.onep.go.th

Documents needed:

- Screening request form (from ONEP)

- Project description (1-5 pages)

- Location map (1:50,000 scale showing site)

- Site plan (1:2,000 scale showing facility layout)

- Environmental context description

- Preliminary environmental assessment

Submission fee: THB 5,000-10,000 (depending on project size)

Processing time: 7-14 days

**Activity 1.3: Receive Screening Decision**

ONEP issues Screening Certificate

Determination: EIA Required, Notification Only, or Exemption

Certificate must be obtained before proceeding with EIA study

If Notification Only: Can proceed with simplified environmental statement (2-4 weeks)

If Full EIA: Proceed to Phase 2

---

## Phase 2: Terms of Reference (TOR) Preparation (Weeks 5-10)

**TOR Purpose:** Scope and framework for the full EIA study

**Activity 2.1: Engage EIA Consultant**

Hire Thai or international firm with EIA experience

Look for:

- ONEP-registered EIA consultant

- Experience with data centers or similar industrial projects

- Track record of ONEP-approved EIA reports

- Bilingual team (Thai/English)

Typical cost: THB 200,000-400,000 for EIA consulting

Major firms: Thai universities, private consulting, international firms

**Activity 2.2: Prepare TOR (Scoping Document)**

TOR defines study scope, methodology, and alternatives

Contents of TOR:

1. **Project Description** (2-3 pages)

- Facility specifications

- Capacity and timeline

- Construction details

- Operations plan

2. **Baseline Environmental Conditions** (2-3 pages)

- Current air quality

- Water quality and resources

- Flora and fauna

- Socioeconomic conditions

- Land use

3. **Potential Environmental Impacts** (3-5 pages)

- Air emissions (generator, vehicles)

- Water consumption and wastewater

- Noise and vibration

- Solid waste

- Ecological impacts

- Socioeconomic impacts

4. **Assessment Methodology** (2-3 pages)

- Data collection methods

- Impact prediction methods

- Significance evaluation criteria

- Specialist involvement

5. **Alternatives Analysis** (2-3 pages)

- Location alternatives

- Technology alternatives

- No-project alternative

- Comparison and recommendation

6. **Mitigation Measures (Preliminary)** (2-3 pages)

- Air quality controls

- Water management

- Noise control

- Waste management

- Community impacts mitigation

7. **Monitoring Plan (Preliminary)** (1-2 pages)

- Post-construction monitoring schedule

- Monitoring parameters

- Mitigation effectiveness verification

**Activity 2.3: Submit TOR to ONEP**

ONEP submission:

- TOR document (20-30 pages, Thai and English)

- Site location map

- Site plan

- Project specifications

- TOR submission fee: THB 5,000-10,000

- Processing time: 3-4 weeks

ONEP review:

- Technical team reviews scope and methodology

- May request TOR modifications

- Issues TOR approval certificate when acceptable

**Activity 2.4: ONEP Provides Comments or Approves TOR**

Possible outcomes:

- **Approved:** Proceed to EIA study

- **Comments:** Revise TOR addressing specific concerns, resubmit

- **Major Revisions Needed:** Complete TOR rewrite and resubmission

**Timeline if Revisions Needed:**

Revisions: 1-2 weeks

Resubmission: 1 week

ONEP review: 2-3 weeks

**Total for revision cycle: 4-6 weeks**

---

## Phase 3: Full EIA Study (Weeks 11-30)

**Duration:** 8-12 weeks of actual study

**Activity 3.1: Baseline Data Collection**

**Air Quality Baseline:**

Conduct 4-week ambient air quality monitoring (at least)

Sample locations: Site location + 4-5 surrounding points

Parameters: PM10, PM2.5, NO₂, SO₂, CO

Equipment: Mobile air quality monitoring station

Cost: THB 50,000-100,000

**Water Quality Baseline:**

If discharging to water body: Conduct water quality sampling

Sample locations: Upstream and downstream of discharge point

Parameters: BOD, COD, TSS, nutrients, pH, temperature

3-4 sampling rounds

Cost: THB 30,000-80,000

**Noise Baseline:**

Baseline noise measurements at site boundaries

Day and night measurements

Seasonal variations (if applicable)

Cost: THB 20,000-40,000

**Ecological Survey (if applicable):**

Flora survey: Plant species and habitats

Fauna survey: Animal species, migration patterns, habitat

Usually 2-3 field surveys

Cost: THB 100,000-200,000 (if detailed study required)

**Socioeconomic Survey:**

Community interviews (50-100 households)

Local impacts assessment

Employment opportunity evaluation

Cost: THB 30,000-60,000

**Activity 3.2: Impact Prediction and Assessment**

**Air Quality Impact Modeling:**

Use EPA models (AERMOD, ISC3) or Thai-approved models

Predict PM, NO₂, SO₂ impacts from:

- Construction activities (dust, vehicles)

- Generator operation (if >300 kW)

- Facility traffic

Comparison to Thai air quality standards

Determination: Will standards be met?

**Water Impact Assessment:**

Water demand vs. availability

Wastewater generation and characteristics

Treatment system performance

Predicted discharge quality vs. standards

Downstream water user impacts

**Noise Impact Assessment:**

Noise sources: Construction, equipment, traffic

Modeling of noise propagation to nearest residents

Comparison to Thai noise standards:

- Daytime: 65 dB(A)

- Nighttime: 55 dB(A)

- Industrial area: May have different standards

Mitigation if standards exceeded

**Waste Management:**

Construction waste: Estimated quantities and disposal

Operational waste: Hazardous (used oil, batteries) and non-hazardous

Waste management procedures and disposal locations

Cost: THB 20,000-50,000

**Traffic and Transportation:**

Construction phase traffic: Vehicles per day, routes

Operational traffic: Delivery vehicles, employee vehicles

Impact on local road network

Mitigation: Traffic management plans

**Ecological Impacts:**

Habitat loss or degradation

Species impacts (direct and indirect)

Cumulative impacts with other projects

Mitigation: Habitat restoration, species protection

**Social and Economic Impacts:**

Employment opportunities

Community services impacts

Economic benefits to local area

Potential displacement or resettlement (if any)

**Activity 3.3: Development of Mitigation Measures**

**Construction Phase Mitigation:**

Dust control: Watering, dust suppression systems, barriers

Noise control: Temporary barriers, equipment silencing, work hour limits

Water management: Erosion control, silt fences

Traffic management: One-way routes, traffic signage

Waste management: Segregation, recycling, proper disposal

Community relations: Notification, regular updates, grievance mechanism

**Operational Phase Mitigation:**

*Air Quality:*

Generator specification: Emission compliance

Regular maintenance: Emission testing, tune-ups

Vegetation/landscaping: Dust settling

Alternative fuels: Natural gas (lower emissions than diesel)

*Water:*

Wastewater treatment system

Water recycling and reuse (cooling tower cycles)

Best management practices for spill prevention

Regular monitoring and testing

*Noise:*

Equipment selection: Low-noise generators, pumps

Enclosures: Sound-insulated enclosures for loud equipment

Preventive maintenance: Regular maintenance reduces noise

Operational controls: Avoid nighttime operations of loud equipment (if possible)

*Waste:*

Hazardous waste: Proper storage, licensed disposal

e-waste: Responsible recycling

Regular maintenance: Minimizes unplanned maintenance waste

*Community:*

Local hiring: Preference to hire local residents

Community outreach: Regular meetings with neighbors

Complaint handling: Mechanism for neighborhood complaints

Support for local causes: Small contribution to local community

**Activity 3.4: Comparative Analysis of Alternatives**

**Alternative 1: Proposed Project**

Capacity: [Specified MW]

Location: [Proposed site]

Environmental impacts: [Detailed assessment]

Costs: [Capital and operational]

Benefits: [Economic, employment, infrastructure]

**Alternative 2: Smaller Capacity (Different Scale)**

Capacity: 50-70% of proposed

Environmental impacts: [Lower impacts expected]

Costs: [Reduced capital, still viable?]

Benefits: [Reduced, but still positive]

Disadvantages: May not meet market needs

**Alternative 3: Different Location (if viable)**

Location: [Alternative site identified]

Environmental impacts: [Comparison]

Costs: [Comparison, may be higher]

Advantages/disadvantages: [Comparative analysis]

**Alternative 4: No Project (Baseline)**

Status quo continuation

No environmental improvement or degradation

No employment or economic benefit

Loss of opportunity for investor

**Recommendation:** Propose original project with specific mitigation measures

**Activity 3.5: Environmental Monitoring Plan (Detailed)**

**Construction Phase Monitoring:**

Air quality: Monthly (not continuous), 4-5 locations

Noise: Monthly at site boundary

Water (if applicable): Monthly for wastewater quality

Waste disposal: Weekly verification of proper disposal

Dust control: Weekly visual inspection

Community complaints: Daily log and response tracking

Duration: Throughout construction period (12-18 months typically)

**Post-Opening Monitoring (Operational Phase):**

*Year 1 (Monthly):*

Air quality: Monthly sampling

Noise: Monthly measurements

Wastewater quality: Monthly testing

Traffic counting: Monthly (weekday and weekend)

Community feedback: Quarterly surveys

Cost: THB 100,000-200,000 for Year 1 monitoring

*Years 2-5 (Quarterly):*

Air quality: Quarterly

Noise: Quarterly

Wastewater: Quarterly

Community feedback: Annually

Cost: THB 50,000-100,000 per year

*Years 6+ (Annual):*

Air quality: Annual

Noise: Annual (unless issues identified)

Wastewater: Annual

Community feedback: As needed

**Monitoring Reports:**

Submitted to ONEP annually (for 5+ years post-opening)

If any exceedances detected: Immediate notification and action

Demonstrates continued compliance with environmental conditions

**Activity 3.6: Preparation of Final EIA Report**

**EIA Report Structure (50-100+ pages typical):**

**Executive Summary (5 pages)**

Project overview

Key findings

Main mitigation measures

Conclusion: Environmentally acceptable with mitigation

**Part 1: Project Description (5-10 pages)**

Background and objectives

Location and site description

Technical specifications

Construction activities and timeline

Operational procedures

Project costs and benefits

**Part 2: Environmental Baseline (10-15 pages)**

Existing air quality

Water resources and quality

Flora and fauna

Soil and geology

Climate and meteorology

Socioeconomic conditions

Land use and development plans

**Part 3: Environmental Impact Assessment (15-25 pages)**

Construction phase impacts

- Air quality (dust, vehicles, equipment)

- Noise and vibration

- Water usage and wastewater

- Solid waste

- Land use changes

- Traffic

- Ecological impacts

- Social impacts

Operational phase impacts

- Air emissions (generator, traffic)

- Noise (equipment, traffic)

- Water consumption and discharge

- Waste generation and management

- Employment and economic impacts

- Community livelihood impacts

Impact significance assessment

- Which impacts are major/minor

- Comparison to Thai standards and guidelines

- Cumulative impacts with other projects

**Part 4: Mitigation and Management Measures (10-15 pages)**

Mitigation during construction

Mitigation during operation

Environmental management plan

Monitoring plan

Contingency measures

Cost estimates for mitigation

**Part 5: Alternatives Analysis (5-10 pages)**

Comparison of project vs. alternatives

Comparison to no-project scenario

Justification for proposed option

**Part 6: Conclusions (2-3 pages)**

Overall environmental acceptability

Conditions of approval

Commitment to monitoring and mitigation

Confidence that standards will be met with mitigation

**Technical Appendices (50+ pages):**

Baseline data tables

Monitoring station locations and results

Model outputs and calculations

Mitigation specifications

Community consultation records

Consultant qualifications

References and data sources

**Total Report Size:** 80-150 pages (EIA report for data center)

**Activity 3.7: Community Consultation (Integrated Throughout)**

**Public Information Disclosure:**

Publicize that EIA is being conducted

Provide information on project in local language

Make draft EIA available for public review (4 weeks minimum)

**Community Meetings:**

Hold meeting in affected community

Explain project and environmental impacts

Receive community concerns and questions

Document comments and responses

Document attendance and issues raised

**Scoping Comments Collection:**

Early scoping meeting with community to identify concerns

Focus on issues important to community

Integrate community concerns into EIA scope

**Benefit-Sharing Discussion:**

Discuss potential benefits to community (employment, infrastructure)

Discuss mitigation for negative impacts

Address community concerns

**Documentation:**

Attendance records (photos, list of attendees)

Minutes of discussions

Community comments (recorded and summarized)

Developer responses to each comment

Include all in final EIA report

---

## Phase 4: ONEP Review and Approval (Weeks 31-40)

**Activity 4.1: Submit Draft EIA to ONEP**

Submission:

- 5+ copies of EIA report (Thai and English)

- Photographs and illustrations

- Documentation of community consultation

- Statement of compliance with TOR

- Project documents supporting appendices

- EIA submission fee: THB 10,000-50,000 (depending on project size)

Review timeline: 4-6 weeks

**Activity 4.2: ONEP Technical Review**

EIA team assigned to review

Technical evaluation of:

- Completeness (all required sections)

- Accuracy of baseline data

- Appropriateness of assessment methodology

- Adequacy of mitigation measures

- Acceptability of monitoring plan

- Conformance with Thai environmental standards

Possible outcomes:

- **Approved:** Proceed to public review

- **Comments:** Developer submits revised sections

- **Major issues:** Entire EIA must be revised

**Activity 4.3: Revisions (if requested)**

Typically 2-4 weeks revision time

Address specific ONEP technical comments

Resubmit revised sections or full report

ONEP re-review: 2-3 weeks

**Activity 4.4: Public Consultation Period (30 days minimum)**

ONEP publicizes EIA availability

Public access to EIA report (at least 30 days)

Public hearing/meeting conducted

ONEP collects public comments

Developer responds to public comments

**Activity 4.5: ONEP Approval Decision**

ONEP committee votes on EIA approval

Committee meeting schedule: Monthly or as needed

Possible outcomes:

- **Approved:** EIA Certificate issued

- **Approved with Conditions:** Specific requirements added

- **Not Approved:** Requires major revision and resubmission

- **Deferred:** Requests additional information or studies

**Approval Conditions Typically Include:**

Specific mitigation measures that MUST be implemented

Monitoring requirements and frequency

Annual reporting to ONEP

Compliance verification procedures

Remedial actions if monitoring shows non-compliance

Community complaint mechanism

Environmental audit requirements

---

# SECTION 3: COSTS AND RESOURCE REQUIREMENTS

## Professional Services Costs

**Cost Variations:**

Complex sites with sensitive ecosystems: Higher (THB 1.5M-2.5M+)

Simple sites in non-sensitive areas: Lower (THB 400K-700K)

International consultant requirements: Higher (20-30% premium)

Multiple alternatives analysis: Higher

## Timeline Summary

**Fast Track (Expedited):** 4-5 months possible if perfect EIA, no major revisions needed

**Delayed (Typical with revisions):** 8-10 months if multiple review rounds

---

# SECTION 4: KEY ENVIRONMENTAL PARAMETERS FOR DATA CENTERS

## Air Quality Standards (Thailand)

**Ambient Air Quality Standards:**

**Data Center Specific:**

Baseline PM10 typically 50-100 µg/m³ in Bangkok area

Generator emissions model may show PM2.5 increase of 5-15 µg/m³ locally

Mitigation needed if exceeds standard

## Water Usage Estimates

**Data Center Water Demand:**

**Baseline:** 5-15 liters per kW per hour (cooling-dependent)

**10 MW Data Center:** Approximately 50,000-150,000 m³ annually

**Percentage of total building water:**

- Cooling: 85-95%

- Domestic use: 5-15%

**Water Source Approval:**

Municipal supply: Straightforward, usually available

Groundwater: Requires extraction permit

Surface water: More complex environmental assessment

Recycled water: Reduces freshwater demand (preferred)

**Wastewater Quality Standards (Discharge to Sewer):**

## Noise Standards

**Thai Industrial Area Noise Standards:**

**Data Center Typical Sources:**

Generator: 75-85 dB (at source, 80 dB at 1m typical)

Cooling towers: 70-80 dB (at source)

HVAC units: 65-75 dB

Traffic: 70-80 dB (on-site vehicles)

**Mitigation:**

Enclosure reduces by 10-20 dB

Distance: Each doubling of distance reduces by 6 dB

Barriers/berms: 5-10 dB reduction

Most sites easily comply in industrial area

## Greenhouse Gas / Carbon Footprint

**Data Center PUE (Power Usage Effectiveness):**

Target: 1.3-1.5 (modern efficient data center)

Baseline: 2.0-2.5 (older, less efficient facilities)

Higher PUE = higher carbon per unit output

**Carbon Footprint Estimation:**

Electricity consumption: [MW × 8,760 hours/year] = MWh/year

Carbon intensity: 450 kg CO₂e per MWh (Thai grid average)

Annual carbon: [MWh × 450 kg] / 1,000 = metric tons CO₂e

**Example (10 MW Data Center):**

10 MW × 8,760 = 87,600 MWh/year

87,600 × 450 kg CO₂e/MWh = 39,420 metric tons CO₂e/year

Renewable energy reduces proportionally

**EIA Requirement:**

Must disclose carbon footprint

Renewable energy plans recommended

Mitigation: Solar panels, wind power, carbon offsets

---

# SECTION 5: COMMON ISSUES AND SOLUTIONS

## Issue 1: EIA Determination - Full vs. Notification Only

**Problem:** Attempting to use simplified notification when full EIA required

**Prevention:**

Get written EIA screening determination from ONEP before starting

Don't assume based on capacity alone

Location sensitivity can trigger full EIA for smaller sites

**Solution if Occurred:**

ONEP will issue correction notice

Must immediately engage EIA consultant

Timeline impact: 6-8 month delay (full EIA instead of 2-4 week notification)

Cannot start construction until full EIA approved

## Issue 2: Inadequate Baseline Data

**Problem:** EIA lacks sufficient baseline environmental measurements

**ONEP Feedback:**

"Air quality monitoring insufficient—need 4+ weeks minimum"

"Insufficient background data on existing conditions"

"Noise survey incomplete—need day and night measurements"

**Prevention:**

Plan baseline monitoring 4-6 months before EIA completion

Use 4+ weeks for air quality

Seasonal variations (if relevant) should be captured

Multiple monitoring locations required

**Solution:**

Conduct additional baseline monitoring (1-2 months)

Resubmit EIA with new data

2-4 week timeline delay

## Issue 3: Inadequate Mitigation Measures

**Problem:** Proposed mitigation is vague or doesn't effectively reduce impacts

**ONEP Comments:**

"Generator enclosure design insufficient"

"Wastewater treatment system not specified"

"Dust control measures are not adequate"

**Prevention:**

Develop specific, detailed mitigation

Include technical specifications and costs

Provide engineering drawings for mitigation systems

Compare to Thai/international best practices

**Solution:**

Revise mitigation section with detailed specifications

Obtain engineering drawings from consultants

Resubmit revised EIA (2-3 week revision period)

1-2 week ONEP re-review

## Issue 4: Community Opposition

**Problem:** Significant community opposition to project during public consultation

**Potential Issues:**

Air pollution concerns

Noise concerns

Water impacts

Loss of agricultural land

Livelihood impacts

**Prevention:**

Proactive community engagement early (during EIA preparation)

Address genuine concerns with genuine mitigation

Demonstrate economic benefits (jobs, taxes)

Clear communication of impacts and controls

**Solution if Occurred:**

Develop specific responses to community concerns

Propose additional mitigation if legitimate concerns

Hold follow-up community meeting

Document responses and community acceptance

ONEP may require additional community consultation

**Note:** Significant unresolved community opposition can delay or prevent approval

## Issue 5: Project Modifications

**Problem:** Project design changes after EIA submitted

**Issue:**

"Capacity increased from 5 MW to 10 MW"

"Location moved to adjacent site"

"Construction timeline changed significantly"

"Water usage will be 50% higher than assessed"

**Prevention:**

Finalize project design before EIA

Avoid major modifications after TOR approval

Get ONEP approval for minor modifications

**Solution:**

If minor (capacity ±10%, similar impacts): Revise EIA section, resubmit

If major (different location, significantly higher capacity): May require new EIA or amended EIA

Timeline impact: 2-4 weeks for minor, 2-3 months for major

## Issue 6: ONEP Requests Additional Studies

**Problem:** ONEP requires specialized environmental study not originally planned

**Example Issues:**

"Archaeological survey required (sensitive area)"

"Geological/seismic assessment required"

"Endangered species assessment required"

"Flood risk modeling required"

**Timeline Impact:** +4-8 weeks per additional study

**Prevention:**

Conduct early environmental site assessment

Review sensitive area maps

Check for protected species on site

Assess flood risk

**Solution:**

Conduct required study immediately

ONEP may grant extension deadline

Integrate into EIA and resubmit

1-2 weeks additional delay

---

# SECTION 6: POST-APPROVAL REQUIREMENTS

## EIA Conditions of Approval

**Typical Conditions:**

1. Implementation of all specified mitigation measures

2. Establishment of environmental management system

3. Annual environmental monitoring (specified parameters and frequency)

4. Annual reporting to ONEP

5. Community notification mechanism

6. Environmental audit (by certified auditor)

7. Specific permit conditions for water, air, hazardous waste

## Environmental Monitoring Plan Implementation

**First Year (Monthly):**

Air quality monitoring (at site boundary and reference locations)

Noise monitoring

Wastewater quality testing (if applicable)

Visual inspection of mitigation systems

Waste disposal verification

Community complaints log

Monthly reports to environmental manager

**Years 2-5 (Quarterly):**

Reduce to quarterly monitoring

Annual reporting to ONEP

If any exceedance found, increased monitoring frequency

**Years 6+ (Annual):**

Annual monitoring (unless issues identified)

Continued reporting to ONEP

**Costs:**

Year 1: THB 100,000-200,000 (monthly monitoring)

Years 2-5: THB 50,000-100,000/year (quarterly)

Years 6+: THB 30,000-60,000/year (annual)

## Non-Compliance Consequences

**Minor Non-Compliance (Missed monitoring, late reporting):**

Written warning from ONEP

Request for corrective action

Opportunity to cure within specified timeline

**Significant Non-Compliance (Exceedance of environmental standards):**

Investigation by ONEP

Notice to comply and remediate

Fines: THB 100,000-500,000+

Project operation restrictions possible

**Severe Non-Compliance (Egregious violation, environmental damage):**

Criminal prosecution possible

Fines: THB 500,000-5,000,000

Imprisonment (responsible parties)

Project closure possible

Environmental remediation required

## Relationship with Other Permits

**Building Permit:** Cannot be issued before EIA approval

**Construction Permit:** Cannot be issued before EIA approval

**Operational Permits:** Water, air, wastewater permits issued after EIA approval

**Critical Path:** EIA must be completed before any construction begins

---

# SECTION 7: CONTACTS AND RESOURCES

## Office of Natural Resources and Environmental Policy & Planning (ONEP)

**Headquarters:**

Address: 60/1 Rama VI Rd, Phayathai, Bangkok 10400

Phone: 0-2535-8800, 0-2535-8811

Fax: 0-2535-8844

Email: [inquiry@onep.go.th](mailto:inquiry@onep.go.th)

Website: www.onep.go.th

Hours: Monday-Friday, 8:30 AM - 4:30 PM

**EIA Division:**

Handles EIA screening, TOR approval, EIA review

Same contact as above (ask for EIA Division)

**Regional ONEP Offices:**

Major provinces have regional offices

Contact info on ONEP website

## Recommended EIA Consultants (Thailand-based)

**International Firms with Thai Operations:**

Jacobs (formerly CH2M Hill)

Tetra Tech

AECOM

ERM (Environmental Resources Management)

**Thai Consulting Firms:**

Thai consultant firms with EIA expertise

University-based research centers

Local environmental consultants (search "EIA consultant Thailand")

**Selection Criteria:**

ONEP registration as EIA consultant

Track record with data center or industrial EIA

Bilingual team

Familiarity with ONEP requirements

**Estimated Consultant Cost Range:**

International firm: THB 800,000-1,500,000

Thai firm: THB 400,000-800,000

University consulting: THB 300,000-600,000

---

# SECTION 8: QUICK REFERENCE - EIA TIMELINE

```

Week 1-4:     Submit screening request → Receive EIA determination

Week 5-10:    Prepare and submit TOR → ONEP review

Week 11-22:   Conduct EIA study (air, water, ecological, social)

Week 23-26:   ONEP technical review and possible revisions

Week 27-30:   Public consultation period (30 days minimum)

Week 31-34:   ONEP committee review and approval

Week 35-40:   Final approval or request for modifications

─────────────────────────────────────────────

TOTAL:        28-40 weeks (6.5-10 months, depending on revisions)

```

**Concurrent Activities (Can run in parallel with EIA):**

BOI application preparation

Land acquisition

Company formation

Financial arrangement

**Sequential to EIA:**

Building permit (cannot apply before EIA approval)

Construction license

Construction start

---

# FINAL CHECKLIST

[ ] Confirm EIA requirement with ONEP (screening)

[ ] Obtain written EIA determination from ONEP

[ ] If full EIA required: Engage EIA consultant

[ ] Develop comprehensive TOR

[ ] Get ONEP TOR approval before starting study

[ ] Conduct 4+ weeks baseline monitoring

[ ] Complete all technical studies (air, water, ecological, etc.)

[ ] Develop detailed mitigation measures

[ ] Prepare draft EIA report (80-150 pages)

[ ] Community consultation and documentation

[ ] Submit EIA to ONEP with all appendices

[ ] Respond to ONEP technical comments

[ ] Support public consultation period

[ ] Obtain written EIA approval from ONEP

[ ] Integrate approval conditions into project plans

[ ] Establish environmental management system

[ ] Plan for annual monitoring and reporting

[ ] Only then proceed with building permit and construction